Verification  Report - 1997 AEAP

Ecotope Inc.

2812 E Madison

Seattle, WA 98112

(206)322-3753

(206)325-7270 (fax)

San Diego Gas & Electric - Study ID 971

1995 Nonresidential New Construction Program – First Year Load Impact Evaluation

Table of Contents

.
.
.
.
.
.
.
.
.

Introduction	1

Program Studied	1

Study Methodology	2

Summary of Findings	2

Recommendation to DRA	2

Data and Documentation Quality	3

Data	3

Documentation	3

Replication and Analysis	3

Sample Design	3

Field Data Collection	4

Simulations	4

Realization Rate Model	4

Net Impact Evaluation	5

Building Categories	6

Replication	7

Modifications to Database and Analytical Procedures	7

Database Modification	7

Analysis Modifications	7

Recommended Changes to Filing Parameters	9

SDG&E – Study ID 971

1995 Nonresidential New Construction

# Introduction

This verification report covers the San Diego Gas & Electric Company’s (SDG&E) Nonresidential New Construction Program for the year 1995.  We are reviewing the first-year load impact evaluation performed by Regional Economic Research, Inc. (RER), VIEWtech, Inc. (VIEWtech) and CIC Research, Inc. (CIC).  This verification is based upon the report submitted to SDG&E on March 5, 1997, referred to herein as “the study”.

Overall, the study is reasonably well-written and well-documented.  There were some problems with the sampling which has the potential of introducing systematic bias into the savings estimates, however these potential problems have been ignored by this verification.  The field data collection and engineering were very detailed and apparently well-executed.  The details are buried in large data files and very extensive input files to a DOE-2 simulation program.  Without completely redoing these simulations it is not possible to fully verify this work.  For the purpose of this verification, we have therefore deemed this part of the study to be fully acceptable.  Therefore, the only issues remaining are whether or not the non-participant sample can be matched to the participant sample, and how the study compares the results from the engineering evaluation of the two samples.

The study over predicts program impacts by approximately 14%.  This is due to the fact that the study did not calibrate the savings by building type.  The study lumped the entire participant and non-participant samples together and predicted program savings based on savings per square foot by energy end use.  This ignores the fact that energy use for different building types can easily vary by a factor of 10.  Furthermore, this difference is not accounted for by using a Title 24 base case, since the lighting requirements under Title 24 vary by a factor of 4 for different building types.  A more appropriate method to compare the participant and non-participant sample is to evaluate savings by building type, as different building types are more or less dominant in the participant and non-participant samples.

## Program Studied

In this utility program, incentives were given to builders who exceeded the levels of energy efficiency mandated in Title 24 in nonresidential new construction.  There were both performance and prescriptive options, however the performance option was utilized by only one building in 1995.  A total of 332 contracts were completed in 1995 for prescriptive measures that increased energy efficiency in lighting, mechanical, glazing and custom measures.

## Study Methodology

The study evaluated energy efficiency in a sample of participant buildings and a matched sample of non-participant buildings, categorized into 14 different use types.  Program savings were assumed to be the increase in overall energy efficiency of the participants compared to the non-participants.  A detailed on-site survey was conducted at each of 253 participant sites and 159 non-participant sites.  The data collected was used to inform DOE-2 building simulations.  Two simulations were performed for each site; one of the building as built, and one of the building just meeting Title 24 standards.  The increase in efficiency of the as-built participant buildings over the Title 24 simulations is the Engineering Estimate of gross savings for the sample.  This estimate of gross savings for the sampled participants was then extended to the total program population by applying the ratio of the utility’s ex ante estimate of the savings for the entire population over the savings for the surveyed sample.  This Engineering Estimate of total gross program savings was further modified by a statistical regression model which predicted the “realization rate”.  This model was informed by billing data, weather data, and end-use metered data.  The net-to-gross analysis was then done using the “simple difference of differences approach” (the gross realized program savings minus the savings of the non-participants over their Title 24 simulations, all normalized by floor area).  This method could effectively take into account freerider and spillover effects as long as the participant and non-participant samples are well matched.  A decision-maker survey was also completed for each site, and a statistical efficiency model was built.  These were used to support the net-to-gross analysis, although they did not directly effect the results of the study.

## Summary of Findings

The data collection, engineering simulations, extension to the total population, and realization rate modeling of the study are all acceptable as presented.  However, the participant and non-participant samples are not well matched.  The participant sample contains a higher proportion of energy intensive buildings than the non-participant sample.  Therefore, the net-to-gross calculations are not acceptable as published.  This verification report has recalculated the net program savings and found them to be about 14% lower than reported by the study.

## Recommendation to DRA

The program savings estimates should be revised as shown in Table 1 below.

Table 1: Verified Total Program Impacts

| Reported Net Energy Savings (KWH) | Verified Net Energy Savings (KWH) | Verification Ratio (energy) | Reported Net Demand Savings (KW) | Verified Net Demand Savings (KW) | Verification Ratio (demand) |
| --- | --- | --- | --- | --- | --- |
| 28,182,426 | 24,247,639 | 0.860 | 4977 | 4282 | 0.860 |

# Data and Documentation Quality

The report is reasonably well documented and the data was in good order.  To fully reproduce all study calculations would have required a monumental verification effort, but this is to be expected from a study of this scope.

## Data

The data files were supplied on CD-ROM, which made handling quite easy.  We made two additional data requests, and they were answered in a reasonable amount of time.

The first data request had a number of different parts and is included in Appendix A.  The response was in the form of letter from the utility, a letter from the primary study author, and a data disk.  (The letters are included in Appendix B).  The disk contained files which showed which measures were incented for each participant.

The second data request is included as Appendix C.  The response was a data file containing the requested information.  A printout of this data is included in Appendix D.

## Documentation

The report was relatively well written and documented.  There were a few aspects of the analysis which were not well documented, but they did not have much to do with the methods which were used in the study.  Rather they arose as questions from the verification.  Those questions which were submitted as data requests were answered in a satisfactory manner.

# Replication and Analysis

### Sample Design

The sample frame for participants was a census of all 285 unique sites participating in the program.  From this census, military and certain other sites requiring security clearances were systematically eliminated.  This left a sample of 279 sites.  An additional 26 sites declined to participate in the study.  This left 253, or 89% of the building sites to make up the participant sample.  This is a very good sample based on the number of buildings, however the buildings that were excluded from the study tended to be the larger sites with greater savings.  The study sample includes 77% of the square foot area covered by the program, and only 67% of the ex ante program savings.  This suggests that the majority of the buildings which were excluded from the study are systematically different from the buildings which were actually studied.  This is somewhat troubling since it indicates that there was effectively no verification of a class of buildings which make up a third of the total program savings.

For the purpose of this verification however, we have accepted the study’s extension of the estimated sample savings to the entire program population.  Based on the ratio of the utility’s ex ante savings estimates, the predicted savings for the sample population was increased by a factor of 1.483.  (See explanation in the study, page 4-15).

The development of the non-participant sample was hindered by the availability of an adequate number of nonparticipating building sites.  The study authors were only able to locate 209 distinct qualified non-participant building sites in the SDG&E service territory.  Of these, only 158 agreed to allow their facilities to be part of the study.  This significantly smaller number of non-participant sites is not ideal, and makes it more difficult to get an accurate estimate of the impacts of participation in the program.  However, it appears that the authors achieved as good a non-participant sample as could be expected under the circumstances.

### Field Data Collection

The study authors appear to have done a very thorough job of surveying and documenting the sample buildings.  We found no problems with the collection or handling of the field data.

### Simulations

The simulations appear to have been carried out correctly and in a manner which adequately addresses potential errors which can evolve from this method of analysis.  The assumptions which are imbedded in the simulation inputs appear to be acceptable.

### Realization Rate Model

An engineering estimate of gross savings was calculated for each building for each of six energy end uses as the Title 24 energy use minus the As-built energy use.  The end use categories identified were: indoor lighting, cooling, heating, ventilation, refrigeration, and process or other.  These savings were summed for the entire participant sample and extended to the entire participant population by the ratio of ex ante savings for the population over ex ante savings for the sample (1.483).  The result is the engineering estimate of gross program savings.  See table 2 below.

Table 2: Simulation estimates of savings and extension to program population.

|  | Estimate for Sample | Estimate for Program |
| --- | --- | --- |
| Number of Participants | 253 | 285 |
| Total Square Footage (ft2) | 9,500,668 | 12,388,871 |
| Total Savings (GWH) | 31.424 | 46.570 |
| Savings per Square Foot (KWH/ft2) | 3.308 | 3.761 |
| Savings per Participant (KWH/site) | 125,190 | 163,405 |

This engineering estimate was adjusted to bring modeled energy use into line with actual billing and weather data.  This was done through the use of a statistically adjusted engineering (SAE) approach to develop calibration factors, referred to here as realization rates, which are applied to the gross program savings for each energy end use.

This statistical model appears to have been carried out in accordance with accepted conventions and with the applicable protocols.  Ultimately, it had the effect of increasing the engineering estimates of savings by slightly more than 2% (a factor of 1.024).

The simulations also developed peak demand fractions for estimating demand impacts.  The realization rates for demand for each end use were assumed to be the same as for total energy use.  When these realization rates were applied, demand savings were increased by slightly less than 2% (a factor of 1.019).

### Net Impact Evaluation

The resulting gross program savings represents all energy savings of the as-built participants over a theoretical group of participant buildings built exactly to the Title 24 standards.  It must be understood that this theoretical group of buildings would never exist, with or without utility programs.  In fact, since Title 24 is a minimum standard, we would expect, on average, that buildings would exceed this mandated level of energy efficiency.  What the study refers to as the gross program savings includes all efficiency increases over the code, whether they incentive based measures or not.  Therefore, to derive an estimate of the actual impact of the program on the energy efficiency of the participant buildings, they must be compared to a matched sample of non-participants.  It can be expected that these non-participants will also exceed the minimum levels mandated by Title 24.

It is critical that the non-participant sample closely matches the participant sample, because the energy efficiency of different types of buildings varies a great deal.  If one sample has a higher percentage of relatively energy intensive buildings, then it will have a higher overall energy use per square foot.  The study failed to take this factor into account in the analysis, and this causes the study to over-predict savings from the program.

The study attempted to take into account some of the variations between building types by accounting for the total energy savings by end use categories.  However, since about 90% of the total program savings are in the lighting end use, this technique has little impact on the results.

Gross program savings estimates per square foot for each energy end use were compared with the results of the non-participant sample.  The difference was assumed to be the actual net program savings per square foot, and the ratio of this to the gross savings per square foot is the implied net-to-gross ratio (NTGR).  See Table 3 below.

Table 3:  Development of Net-to-Gross Ratios for each Energy End Use.

| End Use | Participant Savings (KWH/ft2) | Non-participant Savings (KWH/ft2) | Net-to-Gross Ratio |
| --- | --- | --- | --- |
| Indoor Lighting | 2.816 | 1.319 | 0.532 |
| Cooling | 0.554 | 0.230 | 0.584 |
| Heating | -0.060 | -0.003 | 0.956 |
| Ventilation | 0.065 | 0.008 | 0.880 |
| Refrigeration | 0.303 | 0.002 | 0.995 |
| Process | 0.173 | 0.018 | 0.893 |
| Whole Building | 3.851 | 1.574 | 0.591 |

The NTGRs were then applied to the total gross energy savings, and to gross demand savings to derive the total net program savings.  The resulting savings estimates are shown in Tables 6-7 and 6-8, copied from the study in Appendix A.

### Building Categories

The buildings were divided up into 14 specific building type categories to aid in the comparison of participant and non-participant samples.  To evaluate the adequacy of these samples to predict the efficiency of the participants in the absence of the utility program, we evaluated the balance of building types in each sample.  Tables 2-1 and 2-2 in the study show the building categories used and the number of buildings in each category.  In response to a data request, the utility sent a file which contains the identification number of each building in the study along with a Building Category Code and a written Building Category description.  We were unable to exactly replicate the division of participant and non-participant buildings into the categories listed in the study.  For example, in the non-participant sample, there are 6 buildings which are classified under the Building Category “Supermarket”, and yet Table 2-2 in the study indicates that there are only 3 non-participant “Grocery” buildings.  Since the study did not do their calculations based on building type, this had no impact on their results.  Appendix D includes the summary file sent in response to data request #2.  The column labeled “BldgType” was added by Ecotope to indicate which of the 14 building type categories we used to classify each building for our verification.

The non-participant sample is not well-matched to the participant sample in terms of building type.  The two samples are each dominated by different types of buildings.  For example by floor area, office and retail buildings respectively make up 16% and 24% of the participant sample, while they are only respectively 7% and 13% of the non-participant sample.  On the other hand, the non-participant sample is 15% warehouse, while the participant sample is only 2% warehouse.  Offices and retail buildings tend to have much higher energy densities than warehouses, so this difference in the samples will tend to over-predict total program savings if the samples are not stratified on the basis of building type.

## Replication

An effort was not made to reproduce the results of the DOE-2 simulations done in the study.  This verification reviewed the field data collection methodology and protocol, and the assumptions imbedded in the simulations.  These appeared to be done to accepted standards.  Therefore, the gross engineering estimates were accepted as presented without substantial verification.

A problem was encountered, however, when we evaluated the data sent in response to data request #2.  The totals of floor area and savings did not match those presented in the study by a small amount.  The savings presented are 1.6% less than those provided in response to the data request.  Since we did not have any way to determine which was more accurate, and since the data request was more recent, we used the data received in the data request for our verification.

As noted above, we were unable to replicate the division of the sample buildings into building types as presented in the report.  Since the report did not use these building type categories for any analysis, this did not effect the reported findings.  The assumptions of building type categories used in this verification are shown in Appendix D.

We were able to replicate all other relevant calculations made by the study authors.

# Modifications to Database and Analytical Procedures

This verification modified the study analysis to take into account the different concentration of various building types in the participant and non-participant samples.

## Database Modification

The database was modified based on the data sent in response to data request #2.  The modifications include slight changes in the total square footage and savings from the samples, and reclassification of building sites into 14 building type categories.

## Analysis Modifications

This verification used a simple methodology for accounting for the variation in the two samples (participants and non-participants) on the basis of building type.  Total savings for each building type was calculated for the participants and non-participants and normalized by square foot.  These normalized numbers were then adjusted by the average realization rate developed in the study.  The difference of differences approach was then used to calculate the net savings per square foot for each building type.  This savings fraction was multiplied by the total sample square footage for each building type and summed across building types to derive the net savings from the sample buildings. These calculations are encapsulated in Table 4 below.

Table 4: Verification Calculations for Net Savings from Sample.

The net verified savings from the study sample of 16,350,397 KWH/yr shown in the table above was then modified by the factor 1.483, developed in the study, to extend these savings to the full population.  The total verified program savings are then calculated to be 24,247,639 KWH/yr.  This represents a 14% decrease over the savings reported in the study.  The demand savings could not be separately verified from the data provided; if we assume that the demand savings are reduced by the same amount as the energy savings, then the verified demand savings would be 4.282 MW.

If we assume that the end use splits derived by the study are accurate, then we can report the verified savings on the basis of end use as shown in the table in the section below.

# Recommended Changes to Filing Parameters

Table 5: Verified Total Program Energy and Demand Savings by End Use.

Appendix A: Data Request #1

DATE:	May 15, 1997

TO:	Judy Kelley, SDG&E

FROM:	Dave Baylon, Ecotope Inc.

CC:	Don Schultz, Scott Logan, Joshua Faulk, Fred Sebold, Pat Kirkland

SUBJECT:	Data Request for Study #971 (data request #1)

We have a number of questions regarding study #971, “1995 NRNC First Year Load Impact Evaluation”.  This is a formal data request to enable our verification.

Please send data which reveals the following items:

- The raw data from the site surveys and the inputs to the DOE2 models.

- What measures were incented, for which participants?

- How was non-participant data reconciled with participant data?  How was it assured that there was a good match between the participant and non-participant samples?  Were average savings numbers calculated for each building type?  Normalized by square foot?

- What are the units for hourly numbers in the SITEPRO database?  Are these total usage numbers or savings over Title 24?

- Assumptions for refrigeration equipment for non-participants (p. 4-11) appear to force a realization rate of 1.  How is this justified, and why was the actual refrigeration equipment for the non-participants not determined during the site surveys?

Appendix B: Response to Data Request #1

(Note:  Not available in electronic format, available as hard copy only.  Copy on file at ECONorthwest).

Appendix C:  Data Request #2 and Response

Date: Friday, 06-Jun-97 03:27 PM

From: Gail D. Bennett          \ Internet:    (gbennett@sdge.com)

To:   'Heller@                 \ Internet:    ('heller@mail.eskimo.com)

To:   Ecotope Inc.             \ Internet:    (ecotope@eskimo.com)

cc:   Athena                   \ Internet:    (abesa@sdge.com)

cc:   Besa@                    \ Internet:    (besa@mail.eskimo.com)

cc:   Kirkland@                \ Internet:    (kirkland@mail.eskimo.com)

cc:   Patrick@                 \ Internet:    (patrick@mail.eskimo.com)

Attachment: nrnc_971.xls Code: 03EKP44 \ Created: Unknown [60 Kb]

Subject: FW: Data for Ecotope Data Request #2 for Study #971

Jonathan,

The attached spreadsheet provides the data you requested.  See the attached note

from Bob Ramirez of RER for a few explanatory notes.

If you have any additional questions, please get back to me directly.

Gail

Gail D. Bennett

SDG&E Regulatory Affairs

Phone:  (619) 654-1127

Fax:      (619) 654-0349

Email:   gbennett@sdge.com

>----------

>From:  Bob Ramirez[SMTP:bobram@rer.com]

>Sent:  Thursday, June 05, 1997 2:09 PM

>To:    Bennett, Gail D.

>Cc:    Kirkland, Patrick; Tom Mayer; Fred Sebold; Bob Ramirez

>Subject:       Data for Ecotope Data Request #2 for Study #971

>

>Attached is the data table that will satisfy the following data request from

>John Heller of Ecotope for SDG&E Study ID No. 971.

>

>--------------------------- John Heller's data request

>-------------------------------------------

>Also, it would speed our evaluation if you could provide us with a file which

>summarizes the savings calculated for each building and building type.

>Ideally, this would be in the form of a file which contains the following

>data for each participant and nonparticipant:

>

>* Building ID

>* Building Category (ie. School, small office, restaurant....)

>* Area (ft2)

>* As-built annual energy use (as modeled by DOE2)

>* Title 24 annual energy use

>

>I realize that I have all of this data contained in the various files

>supplied to me already.  However, if you are able to provide this data in a

>timely manner, it would expedite my evaluation.  Please let me know if and

>when you can fill this request.

>

>Thank you,

>

>Jonathan Heller

>

>--------------------------- The Data

>-------------------------------------------

>Attached is the data table you requested from Pat Kirkland which contains the

>following information:

>1) Building ID number (GBID)

>2) Building category code (BldgCatCode)

>3) Building category description (BldgCategory)

>4) Total surveyed floor area (FloorArea)

>5) Whole-building, As-built annual energy use (AsBuiltkWh)

>6) Whole-building, Baseline/Title 24 annual energy use (BasekWh)

>

>The data was derived/calculated from the following files on the "SitePro

>Files & Installation" CD:

>

>LOAD SHAPE LIBRARY FILES (SDGENC\DATABASE directory)

>a) surveyed floor area:  MISC_1.DBF (AsBuilt runs), MISC_B.DBF (Baseline

>runs)

>b) energy intensities:  INT_1.DBF (AsBuilt runs), INT_B.DBF (Baseline runs)

>

>SITE-LEVEL DATABASES (SDGENC\SURVEY\ directory)

>c) Bldg type/code: SITEINFO tables from all *.MDB tables

>

>Hope this helps!

>

>Bob Ramirez (bobram@rer.com)

>Senior Analyst

>RER, Inc.

>

Appendix D:  Data File Sent in Response to Data Request #2 with Assigned Building Types

| GBID | BldgCatCode | BldgCategory | FloorArea | AsBuiltkWh | BasekWh |
| --- | --- | --- | --- | --- | --- |
| RN006 | 052 | Non-refrigerated Warehouse | 10800 | 56381 | 56381 |
| RN007 | 013 | Insurance/Real Estate | 540 | 6700 | 7435 |
| RN010 | 022 | Table Service | 1100 | 31146 | 31146 |
| RN012 | 031 | Supermarket | 14520 | 554798 | 554798 |
| RN016 | 022 | Table Service | 2544 | 18033 | 21115 |
| RN020 | 091 | Church | 11200 | 62933 | 62955 |
| RN021 | 064 | Clinic/Outpatient Care | 1958 | 33340 | 33794 |
| RN023 | 022 | Table Service | 7533 | 454787 | 519936 |
| RN024 | 043 | Other Retail Store | 6600 | 55084 | 73834 |
| RN027 | 014 | Other Office | 1400 | 44775 | 44775 |
| RN033 | 111 | Assembly / Light Mfg. | 24450 | 1938844 | 1945766 |
| RN040 | 012 | Financial / Legal | 1482 | 31389 | 31460 |
| RN041 | 022 | Table Service | 3575 | 277129 | 298858 |
| RN046 | 043 | Other Retail Store | 1925 | 18603 | 24798 |
| RN047 | 011 | Administration & Management | 27534 | 333052 | 404295 |
| RN048 | 022 | Table Service | 6680 | 455110 | 467442 |
| RN049 | 130 | CAR DEALERSHIP | 56675 | 1457441 | 1465623 |
| RN053 | 062 | Nursing Home | 77708 | 673986 | 825225 |
| RN056 | 014 | Other Office | 8100 | 67054 | 67054 |
| RN057 | 052 | Non-refrigerated Warehouse | 80920 | 821647 | 926513 |
| RN058 | 103 | Other Service Shop | 1000 | 113290 | 148351 |
| RN059 | 064 | Clinic/Outpatient Care | 1430 | 17171 | 17171 |
| RN061 | 022 | Table Service | 3850 | 650666 | 656754 |
| RN063 | 043 | Other Retail Store | 3263 | 26738 | 26812 |
| RN067 | 052 | Non-refrigerated Warehouse | 11600 | 35097 | 35126 |
| RN069 | 091 | Church | 26685 | 222047 | 267952 |
| RN070 | 091 | Church | 11160 | 77013 | 126829 |
| RN073 | 032 | Convenience Store | 1452 | 103389 | 103389 |
| RN074 | 031 | Supermarket | 6825 | 297257 | 297257 |
| RN076 | 023 | Bar/Tavern/Nightclub/Other | 1400 | 98248 | 98248 |
| RN079 | 091 | Church | 6754 | 13492 | 14019 |
| RN080 | 092 | Recreational or Other | 63720 | 865318 | 1141407 |
| RN081 | 111 | Assembly / Light Mfg. | 18200 | 1268595 | 1317326 |
| RN089 | 103 | Other Service Shop | 42042 | 347077 | 347077 |
| RN094 | 031 | Supermarket | 18000 | 453095 | 526760 |
| RN106 | 101 | Gas Station/Auto Repair | 1242 | 83622 | 86059 |
| RN107 | 081 | Hotel | 22958 | 265468 | 265468 |
| RN109 | 071 | Daycare or Preschool | 9725 | 156456 | 156456 |
| RN112 | 011 | Administration & Management | 1588 | 48975 | 51820 |
| RN113 | 101 | Gas Station/Auto Repair | 2064 | 102534 | 113106 |
| RN115 | 042 | Shop in Enclosed Mall | 972 | 26949 | 27016 |
| RN116 | 023 | Bar/Tavern/Nightclub/Other | 12900 | 469227 | 515561 |
| RN118 | 091 | Church | 16738 | 55221 | 56994 |
| RN119 | 101 | Gas Station/Auto Repair | 1350 | 256499 | 265172 |
| RN121 | 091 | Church | 20520 | 79746 | 79756 |
| RN123 | 113 | Food/Beverage Processor | 35370 | 2672136 | 2672136 |
| RN130 | 012 | Financial / Legal | 9562 | 135678 | 142156 |
| RN131 | 072 | Elementary / Secondary | 24100 | 79507 | 112763 |
| RN136 | 033 | Other Food Store | 5200 | 24199 | 46022 |
| RN140 | 130 | TESTING LABS/OFFICE | 11500 | 229497 | 210035 |
| RN141 | 014 | Other Office | 50 | 10154 | 10154 |
| RN145 | 043 | Other Retail Store | 4050 | 26818 | 43765 |
| RN146 | 021 | Fast Food or Self Service | 2138 | 65029 | 77107 |
| RN150 | 091 | Church | 21975 | 182778 | 182778 |
| RN153 | 091 | Church | 5875 | 17742 | 20247 |
| RN155 | 031 | Supermarket | 24567 | 1395282 | 1478771 |
| RN159 | 130 | FITNESS GYM AND POOL | 36900 | 362504 | 472099 |
| RN164 | 092 | Recreational or Other | 40753 | 1051482 | 1155649 |
| RN165 | 111 | Assembly / Light Mfg. | 25150 | 773575 | 773575 |
| RN166 | 022 | Table Service | 4430 | 250014 | 250014 |
| RN170 | 111 | Assembly / Light Mfg. | 3375 | 27005 | 27005 |
| RN172 | 064 | Clinic/Outpatient Care | 1200 | 23884 | 23884 |
| RN173 | 013 | Insurance/Real Estate | 1100 | 16918 | 18015 |
| RN178 | 111 | Assembly / Light Mfg. | 20148 | 138232 | 138527 |
| RN183 | 012 | Financial / Legal | 3500 | 98771 | 101328 |
| RN190 | 043 | Other Retail Store | 27400 | 419663 | 526340 |
| RN194 | 091 | Church | 21670 | 144748 | 178513 |
| RN202 | 023 | Bar/Tavern/Nightclub/Other | 4650 | 192241 | 192342 |
| RN204 | 022 | Table Service | 1100 | 66097 | 66598 |
| RN208 | 021 | Fast Food or Self Service | 925 | 143904 | 144048 |
| RN209 | 022 | Table Service | 6138 | 396505 | 405062 |
| RN210 | 101 | Gas Station/Auto Repair | 1450 | 215507 | 215507 |
| RN213 | 092 | Recreational or Other | 14125 | 327830 | 328308 |
| RN214 | 111 | Assembly / Light Mfg. | 7300 | 2760498 | 2771551 |
| RN216 | 021 | Fast Food or Self Service | 2016 | 60037 | 62105 |
| RN217 | 022 | Table Service | 1440 | 117688 | 117688 |
| RN219 | 092 | Recreational or Other | 60125 | 1280914 | 1315151 |
| RN220 | 042 | Shop in Enclosed Mall | 6640 | 95671 | 105417 |
| RN221 | 031 | Supermarket | 82951 | 4428526 | 4428526 |
| RN222 | 042 | Shop in Enclosed Mall | 12100 | 284362 | 309310 |
| RN224 | 092 | Recreational or Other | 6649 | 94970 | 104417 |
| RN225 | 102 | Repair (Non-Auto) | 15750 | 232373 | 232860 |
| RN233 | 022 | Table Service | 3500 | 129505 | 129505 |
| RN234 | 022 | Table Service | 2100 | 118794 | 126041 |
| RN235 | 023 | Bar/Tavern/Nightclub/Other | 4725 | 56218 | 56218 |
| RN238 | 063 | Medical Office | 2800 | 34060 | 34060 |
| RN242 | 052 | Non-refrigerated Warehouse | 22240 | 75170 | 75170 |
| RN247 | 052 | Non-refrigerated Warehouse | 20400 | 160890 | 175859 |
| RN249 | 073 | College or University | 4600 | 73642 | 73642 |
| RN251 | 011 | Administration & Management | 38020 | 440199 | 440199 |
| RN255 | 052 | Non-refrigerated Warehouse | 1360 | 5196 | 5196 |
| RN257 | 022 | Table Service | 1800 | 88771 | 88771 |
| RN259 | 022 | Table Service | 6344 | 159422 | 159422 |
| RN274 | 022 | Table Service | 1250 | 36443 | 38447 |
| RN276 | 081 | Hotel | 12952 | 115010 | 136484 |
| RN279 | 031 | Supermarket | 54160 | 3242578 | 3502718 |
| RN290 | 051 | Refrigerated Warehouse | 19875 | 317653 | 317653 |
| RN292 | 111 | Assembly / Light Mfg. | 12374 | 79319 | 82268 |
| RN294 | 043 | Other Retail Store | 148785 | 2747397 | 3374334 |
| RN296 | 012 | Financial / Legal | 4800 | 99538 | 99538 |
| RN297 | 022 | Table Service | 2070 | 184141 | 188518 |
| RN299 | 011 | Administration & Management | 4016 | 32475 | 35234 |
| RN301 | 111 | Assembly / Light Mfg. | 9960 | 152862 | 156330 |
| RN303 | 083 | Resort | 45207 | 129484 | 146290 |
| RN304 | 130 | FITNESS GYM AND POOL | 10975 | 263887 | 346401 |
| RN306 | 043 | Other Retail Store | 16560 | 259840 | 320503 |
| RN307 | 091 | Church | 10680 | 25719 | 27600 |
| RN309 | 122 | Agriculture | 83920 | 75481 | 79565 |
| RN315 | 043 | Other Retail Store | 1800 | 18909 | 25200 |
| RN317 | 051 | Refrigerated Warehouse | 5184 | 267334 | 267334 |
| RN322 | 014 | Other Office | 1800 | 158134 | 159813 |
| RN324 | 082 | Motel | 4536 | 53297 | 54558 |
| RN325 | 022 | Table Service | 3500 | 145775 | 149979 |
| RN326 | 022 | Table Service | 8710 | 322181 | 322181 |
| RN334 | 042 | Shop in Enclosed Mall | 2268 | 42304 | 42992 |
| RN342 | 022 | Table Service | 11183 | 977735 | 977735 |
| RN343 | 021 | Fast Food or Self Service | 1920 | 60119 | 61922 |
| RN349 | 081 | Hotel | 1500 | 68395 | 70030 |
| RN356 | 130 | MULTI-USE FACILITY (OPERATIONS & OF | 25756 | 953935 | 972612 |
| RN357 | 021 | Fast Food or Self Service | 1875 | 109763 | 111374 |
| RN362 | 022 | Table Service | 1116 | 44757 | 44757 |
| RN376 | 011 | Administration & Management | 5624 | 37695 | 41087 |
| RN377 | 041 | Department/Variety Store | 25220 | 279634 | 385184 |
| RN391 | 052 | Non-refrigerated Warehouse | 41900 | 274000 | 304464 |
| RN392 | 130 | BIO-TECH LAB | 28600 | 946191 | 984667 |
| RN396 | 052 | Non-refrigerated Warehouse | 31616 | 228500 | 237799 |
| RN397 | 091 | Church | 3000 | 80404 | 81640 |
| RN407 | 072 | Elementary / Secondary | 1500 | 1606 | 1943 |
| RN408 | 101 | Gas Station/Auto Repair | 2760 | 353582 | 361567 |
| RN415 | 052 | Non-refrigerated Warehouse | 17981 | 87161 | 89784 |
| RN416 | 022 | Table Service | 1830 | 94865 | 94865 |
| RN425 | 102 | Repair (Non-Auto) | 9000 | 49817 | 58303 |
| RN429 | 052 | Non-refrigerated Warehouse | 9276 | 98769 | 99371 |
| RN434 | 023 | Bar/Tavern/Nightclub/Other | 5400 | 166983 | 167150 |
| RN435 | 083 | Resort | 64430 | 645022 | 676604 |
| RN439 | 043 | Other Retail Store | 4650 | 57861 | 80820 |
| RN440 | 041 | Department/Variety Store | 8250 | 294357 | 314886 |
| RN443 | 043 | Other Retail Store | 12100 | 51718 | 68928 |
| RN444 | 033 | Other Food Store | 1310 | 233726 | 233726 |
| RN448 | 052 | Non-refrigerated Warehouse | 31500 | 28154 | 28662 |
| RN454 | 092 | Recreational or Other | 1980 | 23488 | 29774 |
| RN459 | 043 | Other Retail Store | 2125 | 32967 | 40056 |
| RN462 | 130 | MEDICAL LAB | 22920 | 364932 | 584816 |
| RN464 | 022 | Table Service | 10700 | 268914 | 316988 |
| RN468 | 012 | Financial / Legal | 5100 | 94210 | 94210 |
| RN470 | 043 | Other Retail Store | 5083 | 89675 | 93137 |
| RN471 | 022 | Table Service | 4297 | 129305 | 132935 |
| RN473 | 113 | Food/Beverage Processor | 14350 | 110089 | 112717 |
| RN474 | 043 | Other Retail Store | 7920 | 414764 | 414764 |
| RN476 | 091 | Church | 23200 | 121151 | 136579 |
| RN479 | 052 | Non-refrigerated Warehouse | 50800 | 195988 | 203235 |
| RN481 | 072 | Elementary / Secondary | 26360 | 84603 | 135913 |
| RN702 | 063 | Medical Office | 41536 | 562800 | 562800 |
| RN705 | 092 | Recreational or Other | 34300 | 670611 | 671153 |
| RN722 | 043 | Other Retail Store | 588 | 7697 | 8487 |
| RN737 | 101 | Gas Station/Auto Repair | 4600 | 77249 | 81752 |
| RN738 | 043 | Other Retail Store | 6900 | 81218 | 90166 |
| RN741 | 032 | Convenience Store | 3000 | 78744 | 78744 |
| RP002 | 092 | Recreational or Other | 16384 | 192976 | 279767 |
| RP003 | 012 | Financial / Legal | 8526 | 57497 | 79686 |
| RP004 | 011 | Administration & Management | 63876 | 775570 | 959720 |
| RP005 | 130 | WATER FILTRATION PLANT | 22800 | 772533 | 817359 |
| RP006 | 072 | Elementary / Secondary | 31800 | 221893 | 290961 |
| RP007 | 012 | Financial / Legal | 45382 | 408045 | 588897 |
| RP008 | 043 | Other Retail Store | 16500 | 187369 | 244212 |
| RP009 | 072 | Elementary / Secondary | 59755 | 442594 | 485772 |
| RP010 | 064 | Clinic/Outpatient Care | 5622 | 101648 | 103202 |
| RP012 | 072 | Elementary / Secondary | 30714 | 429095 | 513152 |
| RP013 | 014 | Other Office | 50358 | 1356254 | 1604365 |
| RP015 | 111 | Assembly / Light Mfg. | 13800 | 754373 | 1262902 |
| RP016 | 012 | Financial / Legal | 9600 | 99117 | 113582 |
| RP017 | 041 | Department/Variety Store | 135000 | 2416140 | 3198056 |
| RP018 | 072 | Elementary / Secondary | 43000 | 227909 | 304806 |
| RP019 | 012 | Financial / Legal | 23612 | 317171 | 349891 |
| RP020 | 031 | Supermarket | 45700 | 2725084 | 2886283 |
| RP021 | 012 | Financial / Legal | 33217 | 288396 | 422892 |
| RP022 | 011 | Administration & Management | 21720 | 113679 | 158838 |
| RP025 | 111 | Assembly / Light Mfg. | 8444 | 97466 | 122441 |
| RP026 | 011 | Administration & Management | 64146 | 954271 | 1134698 |
| RP027 | 072 | Elementary / Secondary | 31530 | 236496 | 344647 |
| RP028 | 014 | Other Office | 56209 | 430809 | 646291 |
| RP029 | 043 | Other Retail Store | 7318 | 45326 | 54141 |
| RP030 | 011 | Administration & Management | 31225 | 502248 | 786359 |
| RP031 | 052 | Non-refrigerated Warehouse | 25000 | 144881 | 182215 |
| RP032 | 043 | Other Retail Store | 10890 | 65209 | 73694 |
| RP033 | 072 | Elementary / Secondary | 36949 | 562743 | 683608 |
| RP034 | 092 | LIBRARY | 4803 | 50389 | 66748 |
| RP035 | 072 | Elementary / Secondary | 32322 | 185804 | 277096 |
| RP036 | 092 | Recreational or Other | 48620 | 450912 | 670959 |
| RP037 | 043 | Other Retail Store | 9120 | 514709 | 564681 |
| RP038 | 022 | Table Service | 3022 | 285758 | 304041 |
| RP039 | 041 | Department/Variety Store | 31200 | 267600 | 373725 |
| RP040 | 032 | Convenience Store | 2400 | 285674 | 309854 |
| RP041 | 043 | Other Retail Store | 3444 | 88390 | 109428 |
| RP042 | 032 | Convenience Store | 1858 | 261355 | 276385 |
| RP043 | 072 | Elementary / Secondary | 63630 | 613542 | 762805 |
| RP044 | 031 | Supermarket | 32520 | 940502 | 1117004 |
| RP045 | 092 | Recreational or Other | 7042 | 67220 | 73243 |
| RP046 | 072 | Elementary / Secondary | 77117 | 442994 | 683959 |
| RP047 | 021 | Fast Food or Self Service | 4320 | 333265 | 339763 |
| RP048 | 013 | Insurance/Real Estate | 38500 | 850107 | 946287 |
| RP049 | 012 | Financial / Legal | 30206 | 108740 | 181202 |
| RP050 | 061 | Hospital | 1600 | 25237 | 42831 |
| RP051 | 072 | Elementary / Secondary | 33786 | 200200 | 253913 |
| RP052 | 063 | Medical Office | 26784 | 227049 | 259039 |
| RP053 | 130 | FIREHOUSE | 7566 | 66428 | 84089 |
| RP054 | 072 | Elementary / Secondary | 42396 | 268036 | 363135 |
| RP055 | 072 | Elementary / Secondary | 54515 | 382193 | 464740 |
| RP056 | 041 | Department/Variety Store | 150196 | 3182013 | 3956849 |
| RP057 | 021 | Fast Food or Self Service | 2979 | 157726 | 170646 |
| RP058 | 092 | Recreational or Other | 33960 | 527679 | 725333 |
| RP059 | 072 | Elementary / Secondary | 64056 | 514900 | 537863 |
| RP060 | 081 | Hotel | 14200 | 348194 | 369535 |
| RP062 | 072 | Elementary / Secondary | 1360 | 31603 | 35256 |
| RP063 | 014 | Other Office | 2368 | 12893 | 13878 |
| RP064 | 113 | Food/Beverage Processor | 9639 | 231024 | 245775 |
| RP065 | 130 | FIREHOUSE | 71050 | 1278741 | 1400657 |
| RP066 | 072 | Elementary / Secondary | 35000 | 236363 | 254254 |
| RP067 | 072 | Elementary / Secondary | 38630 | 171226 | 258994 |
| RP068 | 041 | Department/Variety Store | 36100 | 430735 | 618386 |
| RP069 | 022 | Table Service | 6084 | 299179 | 299948 |
| RP070 | 130 | PUMP STATION | 7700 | 1178110 | 2147325 |
| RP071 | 072 | Elementary / Secondary | 34095 | 326978 | 409698 |
| RP072 | 011 | Administration & Management | 2601 | 27180 | 27699 |
| RP073 | 111 | Assembly / Light Mfg. | 18358 | 157829 | 232558 |
| RP074 | 073 | College or University | 6500 | 101054 | 140598 |
| RP075 | 022 | Table Service | 6840 | 373887 | 378695 |
| RP076 | 073 | College or University | 103500 | 1241241 | 2032724 |
| RP077 | 072 | Elementary / Secondary | 31320 | 132612 | 205149 |
| RP078 | 072 | Elementary / Secondary | 38329 | 261498 | 388313 |
| RP079 | 072 | Elementary / Secondary | 31650 | 320295 | 405020 |
| RP080 | 031 | Supermarket | 14280 | 249946 | 287355 |
| RP081 | 011 | Administration & Management | 11000 | 122000 | 151655 |
| RP082 | 111 | Assembly / Light Mfg. | 1500 | 163597 | 166575 |
| RP085 | 064 | Clinic/Outpatient Care | 6183 | 55690 | 70245 |
| RP086 | 043 | Other Retail Store | 28716 | 370253 | 480443 |
| RP088 | 081 | Hotel | 65736 | 751178 | 820312 |
| RP089 | 091 | Church | 3760 | 22114 | 25257 |
| RP090 | 014 | CRIME LAB/SHERIFF OFFICE | 101470 | 1732907 | 1836328 |
| RP091 | 072 | Elementary / Secondary | 67548 | 270732 | 380861 |
| RP092 | 072 | Elementary / Secondary | 31240 | 250614 | 304922 |
| RP093 | 092 | Recreational or Other | 2796 | 21979 | 37233 |
| RP094 | 103 | Other Service Shop | 87770 | 1398568 | 1447260 |
| RP095 | 022 | Table Service | 4590 | 387115 | 394124 |
| RP096 | 130 | WATER RECLAMATION PLANT | 31260 | 746442 | 799033 |
| RP098 | 072 | Elementary / Secondary | 38419 | 242914 | 375130 |
| RP099 | 072 | Elementary / Secondary | 37600 | 101452 | 190887 |
| RP100 | 072 | Elementary / Secondary | 218500 | 827737 | 1403144 |
| RP101 | 072 | Elementary / Secondary | 43764 | 305742 | 422742 |
| RP102 | 091 | Church | 19125 | 123906 | 141903 |
| RP103 | 022 | Table Service | 4536 | 382874 | 395540 |
| RP104 | 072 | Elementary / Secondary | 37067 | 296092 | 370906 |
| RP105 | 072 | Elementary / Secondary | 30550 | 265977 | 317220 |
| RP107 | 073 | College or University | 48600 | 641428 | 670948 |
| RP108 | 043 | Other Retail Store | 22000 | 381563 | 472349 |
| RP109 | 092 | Recreational or Other | 39298 | 1599842 | 1636619 |
| RP110 | 091 | Church | 19800 | 100301 | 129547 |
| RP111 | 072 | Elementary / Secondary | 30200 | 164252 | 246390 |
| RP112 | 021 | Fast Food or Self Service | 4560 | 171383 | 176182 |
| RP113 | 012 | Financial / Legal | 7196 | 197134 | 203451 |
| RP114 | 021 | Fast Food or Self Service | 3136 | 219402 | 227809 |
| RP115 | 043 | Other Retail Store | 7050 | 189118 | 204130 |
| RP116 | 111 | Assembly / Light Mfg. | 3950 | 46090 | 51446 |
| RP119 | 130 | BIO-TECH LAB | 30700 | 1840566 | 1989654 |
| RP121 | 033 | Other Food Store | 10296 | 246529 | 300194 |
| RP122 | 031 | Supermarket | 48860 | 2141713 | 2680735 |
| RP124 | 082 | Motel | 22760 | 121545 | 123368 |
| RP125 | 130 | HAIR & NAIL SALON | 1275 | 13931 | 15818 |
| RP126 | 043 | Other Retail Store | 5300 | 39661 | 48655 |
| RP127 | 021 | Fast Food or Self Service | 3567 | 330015 | 335270 |
| RP128 | 051 | Refrigerated Warehouse | 10400 | 514028 | 538988 |
| RP129 | 072 | Elementary / Secondary | 25413 | 223308 | 235704 |
| RP130 | 072 | Elementary / Secondary | 59220 | 447000 | 507806 |
| RP131 | 072 | Elementary / Secondary | 37404 | 245240 | 333468 |
| RP132 | 011 | Administration & Management | 50000 | 301705 | 397903 |
| RP133 | 031 | Supermarket | 62810 | 3667049 | 4367658 |
| RP134 | 063 | Medical Office | 6300 | 93940 | 110182 |
| RP135 | 072 | Elementary / Secondary | 41194 | 324509 | 481121 |
| RP136 | 061 | Hospital | 19896 | 559009 | 668436 |
| RP137 | 041 | Department/Variety Store | 341700 | 4089373 | 4091543 |
| RP138 | 012 | Financial / Legal | 12100 | 375716 | 385037 |
| RP139 | 130 | RESTAURANT/OUTPATIENT CLINIC/RESIDE | 48093 | 1247441 | 1317089 |
| RP140 | 130 | WORK FURLOUGH RESIDENCE | 26560 | 292293 | 373837 |
| RP141 | 021 | Fast Food or Self Service | 3600 | 269290 | 281192 |
| RP142 | 111 | Assembly / Light Mfg. | 3200 | 18705 | 19897 |
| RP143 | 111 | Assembly / Light Mfg. | 24000 | 276233 | 308102 |
| RP144 | 021 | Fast Food or Self Service | 5625 | 336321 | 365204 |
| RP146 | 032 | Convenience Store | 1858 | 255254 | 271855 |
| RP148 | 011 | Administration & Management | 42008 | 751641 | 865855 |
| RP149 | 043 | Other Retail Store | 16800 | 334056 | 394409 |
| RP150 | 011 | Administration & Management | 70000 | 2003386 | 2041215 |
| RP151 | 111 | Assembly / Light Mfg. | 14370 | 208034 | 214498 |
| RP152 | 043 | Other Retail Store | 18000 | 327633 | 383298 |
| RP153 | 031 | Supermarket | 53400 | 3529625 | 3786804 |
| RP155 | 012 | Financial / Legal | 13500 | 177603 | 195705 |
| RP157 | 111 | Assembly / Light Mfg. | 108360 | 3223637 | 3267648 |
| RP159 | 111 | Assembly / Light Mfg. | 27200 | 347522 | 362199 |
| RP160 | 013 | Insurance/Real Estate | 25360 | 274297 | 364234 |
| RP161 | 111 | Assembly / Light Mfg. | 16014 | 167086 | 199758 |
| RP163 | 043 | Other Retail Store | 39790 | 904823 | 934240 |
| RP164 | 073 | College or University | 30178 | 349930 | 379639 |
| RP165 | 072 | Elementary / Secondary | 41240 | 277965 | 323126 |
| RP166 | 092 | Recreational or Other | 33042 | 325027 | 387707 |
| RP167 | 091 | Church | 2600 | 14521 | 15451 |
| RP168 | 130 | RESEARCH INSTITUTE | 57700 | 2522785 | 3089115 |
| RP169 | 021 | Fast Food or Self Service | 7314 | 234827 | 247453 |
| RP170 | 111 | Assembly / Light Mfg. | 59550 | 242898 | 430962 |
| RP171 | 031 | Supermarket | 65460 | 4047284 | 4257746 |
| RP172 | 043 | Other Retail Store | 25708 | 311404 | 504843 |
| RP173 | 043 | Other Retail Store | 97724 | 2203692 | 2380862 |
| RP174 | 052 | Non-refrigerated Warehouse | 134011 | 140041 | 202740 |
| RP175 | 043 | Other Retail Store | 107118 | 2378387 | 2637281 |
| RP176 | 072 | Elementary / Secondary | 99720 | 511707 | 655774 |
| RP177 | 031 | Supermarket | 55400 | 2921213 | 3169620 |
| RP178 | 022 | Table Service | 5796 | 293458 | 299062 |
| RP179 | 021 | Fast Food or Self Service | 3762 | 282880 | 290806 |
| RP180 | 043 | Other Retail Store | 20122 | 283388 | 378534 |
| RP181 | 041 | Department/Variety Store | 126489 | 2664683 | 3100335 |
| RP182 | 022 | Table Service | 4900 | 396512 | 410057 |
| RP183 | 041 | Department/Variety Store | 127911 | 2181818 | 3076051 |
| RP184 | 021 | Fast Food or Self Service | 11025 | 593064 | 643489 |
| RP185 | 103 | Other Service Shop | 4700 | 142460 | 150510 |
| RP186 | 031 | Supermarket | 50350 | 1834673 | 2412547 |
| RP187 | 021 | Fast Food or Self Service | 5880 | 277903 | 301400 |
| RP188 | 043 | Other Retail Store | 27691 | 853734 | 917975 |
| RP189 | 014 | Other Office | 1440 | 20622 | 24551 |
| RP190 | 021 | Fast Food or Self Service | 14760 | 757826 | 827637 |
| RP191 | 031 | Supermarket | 50107 | 2699564 | 2699641 |
| RP192 | 021 | Fast Food or Self Service | 3402 | 258948 | 263112 |
| RP193 | 011 | Administration & Management | 29790 | 364072 | 448029 |
| RP194 | 021 | Fast Food or Self Service | 3100 | 267845 | 272587 |
| RP195 | 111 | Assembly / Light Mfg. | 100600 | 967872 | 1013536 |
| RP196 | 130 | BIO-TECH LAB | 43601 | 1709670 | 1891383 |
| RP197 | 072 | Elementary / Secondary | 65230 | 443115 | 544562 |
| RP198 | 111 | Assembly / Light Mfg. | 45830 | 525171 | 673916 |
| RP199 | 063 | Medical Office | 7181 | 140718 | 170590 |
| RP200 | 022 | Table Service | 4996 | 402770 | 418404 |
| RP201 | 022 | Table Service | 5000 | 276825 | 299729 |
| RP202 | 043 | Other Retail Store | 6808 | 161044 | 189849 |
| RP203 | 043 | Other Retail Store | 11340 | 248837 | 389850 |
| RP204 | 031 | Supermarket | 52791 | 2776689 | 3108935 |
| RP205 | 043 | Other Retail Store | 6254 | 198147 | 236936 |
| RP206 | 041 | Department/Variety Store | 119423 | 2057688 | 2530993 |
| RP207 | 011 | Administration & Management | 42390 | 576164 | 665353 |
| RP208 | 043 | Other Retail Store | 8100 | 167039 | 224626 |
| RP209 | 011 | Administration & Management | 35306 | 259945 | 321008 |
| RP210 | 072 | Elementary / Secondary | 38016 | 127962 | 200816 |
| RP211 | 043 | Other Retail Store | 18000 | 85731 | 146114 |
| RP212 | 043 | Other Retail Store | 34959 | 464854 | 534722 |
| RP213 | 052 | Non-refrigerated Warehouse | 50850 | 633719 | 703775 |
| RP214 | 111 | Assembly / Light Mfg. | 20724 | 173786 | 253725 |
| RP215 | 031 | Supermarket | 52000 | 2699808 | 3305022 |
| RP216 | 111 | Assembly / Light Mfg. | 90336 | 4167500 | 4592729 |
| RP217 | 021 | Fast Food or Self Service | 10600 | 743954 | 752508 |
| RP218 | 043 | Other Retail Store | 183157 | 3629401 | 3891273 |
| RP219 | 031 | Supermarket | 63635 | 2882552 | 3053566 |
| RP220 | 043 | Other Retail Store | 19493 | 407648 | 414226 |
| RP221 | 041 | Department/Variety Store | 30400 | 351497 | 452757 |
| RP223 | 022 | Table Service | 6528 | 370370 | 377270 |
| RP224 | 043 | Other Retail Store | 8075 | 71413 | 113503 |
| RP225 | 031 | Supermarket | 58050 | 2995875 | 3397875 |
| RP226 | 031 | Supermarket | 58000 | 3235423 | 3913432 |
| RP227 | 031 | Supermarket | 62658 | 2982319 | 3536913 |
| RP228 | 071 | Daycare or Preschool | 11800 | 131343 | 155628 |
| RP229 | 021 | Fast Food or Self Service | 3150 | 261975 | 268718 |
| RP230 | 092 | Recreational or Other | 17808 | 172725 | 261020 |
| RP231 | 043 | Other Retail Store | 19764 | 414666 | 502521 |
| RP232 | 043 | Other Retail Store | 16500 | 505254 | 535181 |
| RP233 | 031 | Supermarket | 62378 | 3917566 | 4265863 |
| RP234 | 041 | Department/Variety Store | 132798 | 2365653 | 3270576 |
| RP235 | 041 | Department/Variety Store | 27784 | 441570 | 571103 |
| RP236 | 043 | Other Retail Store | 46712 | 700281 | 713368 |
| RP237 | 043 | Other Retail Store | 33600 | 621617 | 632795 |
| RP238 | 041 | Department/Variety Store | 115877 | 2018390 | 2516801 |
| RP239 | 043 | Other Retail Store | 26721 | 528016 | 574642 |
| RP240 | 014 | Other Office | 41625 | 511741 | 567142 |
| RP241 | 031 | Supermarket | 45635 | 2017242 | 2285744 |
| RP242 | 092 | Recreational or Other | 80224 | 1279686 | 1421092 |
| RP248 | 130 | AIRPORT | 230900 | 7089854 | 8209017 |
| RP252 | 011 | Administration & Management | 10240 | 117367 | 144185 |
| RP254 | 011 | Administration & Management | 20192 | 210678 | 267821 |
| RP255 | 011 | Administration & Management | 11790 | 84500 | 102632 |
| RP257 | 111 | Assembly / Light Mfg. | 52642 | 1437561 | 1629309 |
| RP258 | 014 | Other Office | 51757 | 823999 | 968368 |
| RP261 | 062 | Nursing Home | 12135 | 236753 | 305582 |
| RP262 | 011 | Administration & Management | 23866 | 290436 | 324782 |
| RP266 | 011 | Administration & Management | 181430 | 3392304 | 3685482 |
| RP270 | 091 | Church | 13260 | 69252 | 76155 |
| RP272 | 043 | Other Retail Store | 12150 | 249529 | 311192 |
| RP274 | 072 | Elementary / Secondary | 43300 | 342632 | 404201 |
| RP275 | 091 | Church | 7969 | 60865 | 80208 |
| RP500 | 014 | Other Office | 2925 | 28706 | 31159 |
| RP501 | 092 | Recreational or Other | 4035 | 139854 | 144170 |
| RP530 | 073 | College or University | 19512 | 316885 | 323069 |
| RP531 | 073 | College or University | 18771 | 372877 | 395998 |
| RP532 | 073 | College or University | 12100 | 193073 | 278550 |
| RP533 | 073 | College or University | 53700 | 696578 | 1012179 |
| RP540 | 011 | Administration & Management | 70891 | 1022038 | 1102648 |
| RP541 | 073 | College or University | 133980 | 4995226 | 5426954 |
| RP542 | 071 | Daycare or Preschool | 5216 | 41101 | 49903 |
| RP543 | 073 | College or University | 52557 | 999575 | 1158306 |
| RP544 | 130 | CENTRAL MECHANICAL PLANT | 20000 | 4443872 | 5088051 |
| RP550 | 073 | College or University | 163823 | 2054603 | 2580183 |
| RP551 | 073 | College or University | 26550 | 265504 | 281445 |
| RP560 | 011 | Administration & Management | 70970 | 1998510 | 2459052 |
| RP590 | 111 | Assembly / Light Mfg. | 80072 | 879611 | 971036 |
| RP600 | 011 | Administration & Management | 13600 | 226367 | 261180 |
| RP601 | 011 | Administration & Management | 5080 | 234131 | 271535 |
| RP602 | 011 | Administration & Management | 42740 | 444546 | 545821 |
| RP603 | 021 | Fast Food or Self Service | 2950 | 337967 | 345960 |
| RP604 | 061 | Hospital | 20172 | 515436 | 560395 |