- MEMO

| To: | Don Schultz, CPUC/ORA |
| --- | --- |
| From: | Kenneth M. Keating, ORA Evaluation Consultant |
| Date: | August 8, 1998 |
| Subject: | Review Memo for PG&E 389: NRNC – Whole Building |

- REVIEW SUMMARY

1. Utility:  Pacific Gas and Electric Company                        			Study ID: 389

Program and PY:  Non-Residential New Construction  Program:  PY1996

- End Use(s):  Whole Building

2.  Utility Study Title:  “Impact Evaluation of Pacific Gas and Electric Company’s  1996 Non-Residential New Construction Program ”

3. Type of Study:  1st Year Load Impact Study                		 Required by Table 8A: Yes.

- 4. Applicable Protocols: Tables 5, 6, 7, and C-8

Study Completion:  March 1, 1998 	Required Documentation Received:   Yes

Retroactive Waivers:   Retroactive Waiver approved on September 24, 1997 that allows (a) the study to determine the appropriate sample size by making the precision estimates around the load impact estimates, (b) the use of short-term metering in place of whole building billing data to calibrate the engineering models  and (c) the use of two different methodologies to estimate net load impacts, with the selection of the filed method based on the lowest error bound around the “net kWh savings.”

- 5.  Reported Impact Results:

- Average Annual Gross Load Impacts.

Whole building: peak: 20,000 kW (0.00137 kW per unit [kW/sq.-ft/yr]; 1.046 gross realization rate).  Energy:  83,970,000 kWh (5.75 kWh per unit[kWh/sq.-ft/yr];  1.044 gross realization rate).

- Average Annual  Net Load Impacts:

Whole building:  peak: 15,600 kW (0.00107 kW per unit [kW/sq.-ft/yr];  0.816 net demand realization rate).  Energy:  63,229,410 kWh (4.33 kWh per unit [kWh/sq.-ft/yr];  0.786 net energy realization rate).

- Net-to-gross ratios:  Peak:  	0.78

Energy:	0.753

- 7.  Review Findings:

- Conformity with Protocols:  The study is generally in conformity with the protocols as modified by the retroactive waiver.

- Acceptability of Study results: This study will have a Verification Report.  The main problem is the large amount of spillover claimed.

Recommendations:  The recommendation is to accept the earnings claims as documented in this Study and laid out in Table 6, with the exception of the claims for spillover as laid out in the report.  This will result in a net load impact and earnings reduction for peak of 25.5% and energy of 19.4%.

- OVERVIEW

The Non Residential New Construction Program is a shared savings program for purposes of shareholder incentives.  As such, the actual ex post evaluation results from the first year load impact study are important to the calculation of the shareholder incentive.  Approximately 24% of the shared savings shareholder incentives for the PG&E are dependent on this NRNC study, or $7.4 million.   More than 20% of the 2nd year earnings claim, or about $1.0 million, is based on an untested methodology to estimate program spillover.

#### REPORTED IMPACT RESULTS

- Average Annual Gross Load Impacts.

Whole building: peak: 20,000 kW (0.00137 per unit [kW/sq.-ft/yr]; 1.046 gross realization rate).  Energy:  83,970,000 kWh (5.75 kWh per unit[kWh/sq.-ft/yr];  1.044 gross realization rate).

- Average Annual  Net Load Impacts:

Whole building:  peak: 15,600 kW (0.00107 kW per unit [kW/sq.-ft/yr];  0.816 net demand realization rate).  Energy:  63,229,410 kWh (4.33 kWh per unit [kWh/sq.-ft/yr];  0.786 net energy realization rate).

- Net-to-gross ratios:  Peak:  	0.78

Energy:	0.753

#### ASSESSMENT OF STUDY METHODOLOGY AND RESULTS

The Study estimated the gross load impacts of both participants and a matched set of nonparticipants whose buildings were completed in the same year as those of the participants.  For the gross load impacts, there were 138 participants and 138 nonparticipants.  Each sample point was visited and the on-site auditors collected (a) building characteristics, (b) as-found operating schedules, and (c) end-use metered parameters.  The as-found, as-operated building was modeled with DOE 2.1E software to normalize consumption to long term weather, account for interactions among measures and end-uses, and to provide a baseline simulation for each building, participant or not.  The baseline took into account the required minimum Title 24 efficiencies, but without the Title 24 operating assumptions.  The end-use metered data or billing data were sometimes used to calibrate the assumptions and/or reported information that otherwise would have gone into the simulations.  Actual, as-operated conditions in the buildings were used.  In this way gross load impacts, above the baseline conditions could be calculated for each building.  Nonparticipant buildings had some efficiencies above Title 24 -- almost half as much efficiency gain as the gross simulated impacts of the participant buildings.

In line with the requested and approved retroactive waiver, the study contractors approximated the net load impacts of the program using two approaches.  The first was to compare the gross load impacts of the nonparticipant sample that was closely matched to the participant sample, with those of the participants.  This is basically a “difference of differences” approach.  It assumes that the nonparticipants reflect current practice, which would have been mimicked in the absence of the program, or perhaps improved upon due to the self-selection bias among participants.  The assumption is that the participants wouldn’t have built any less efficiently than the nonparticipants did.  Any effect due to the self-selection of the participants is potentially off-set by potential changes in the behavior of the nonparticipants due to the existence of the program, but neither effect is measured or isolated.

The second approach is to perform an econometric modeling exercise by which variables that may be related to the choice of efficiency options are tested and built into a model that may explain some of the other unmatched attributes of the participants or non-participants, or the unobserved differences (Mills ratio).  Its strength is that it can consider differences that are not captured in direct comparisons.  Its weaknesses include its dependence on self-reported attitudes, behavior, and constructed variables.  Its flexibility allowing the analyst to construct,  test, and select variables is both a strength and a weakness in adversarial situations.  By adding and subtracting variables in order to improve precision, it generally will provide a more precise estimate than a “difference of differences” approach which is less “flexible.”

The retroactive waiver required that the selection of results be based on the model “that yields the lower error bound….computed as the standard deviation of the net kWh savings (rather than the percent savings from Title 24) at the program level.”  In this case, the net kWh results from the “difference of differences” approach was 39,054 MWh, plus or minus 25.5% (page 63), and the result from the econometric approach was 63,229 MWh plus or minus 19.5% (page 64).  Assuming that all true variance is captured in the modeling and the components are correctly operationalized, the Company apparently selected the results from the econometric approach to net-to-gross that was the larger net savings, in line with the retroactive waiver.

Evaluation Issues:

This appears to be a strong load impact study in terms of its gross load impact analysis. This study’s net results depend to an extraordinary amount on a new “scaled variable” approach to estimating spillover.  Previous studies had been unable to identify any program spillover on nonparticipants.  For this study, fully 26% of the load impacts claimed for purposes of shareholder incentives are from spillover.  There are several problems with the approach to estimating spillover.

- There is no explanation in the text of the study or in the appendix about how the scales were constructed, why a seven point scale was used, or whether the questions produced reliable and internally consistent results.  The rigorous development of a scaled variable is not documented.

- The artificially constructed scaled variable is at best an ordinal variable, but it is being used in the simplified OLS regression, which assumes nominal or interval variables.

- The construct of the variable is logically biased in that there is only one “no-influence” response available for nonparticipants to select out of the 7 responses presented.  All the others responses always add more spillover.

- The construct of the scaled variable will tend to bias the response in that it is clear that the interviewer seeks some gradient of influence, and the response bias is to give some “influence” which will always be more than zero.

- The no-influence response is at the extreme end of a seven point scale.  Research has indicated that when confronted with a wide set of gradations, respondents hesitate to choose an extreme response.

- The study contractor argues that the 7-point scale is more sensitive than a binary variable in identifying the subtle aspects of spillover (page 93).  Without documentation to indicate that a 7 point scale was the optimal for getting at nuances of real influences, we might assume that a 15 point scale would have been even better at eliciting spillover.  (And given the logic of point 5, it clearly would have elicited more spillover).

Although the Company has selected the model results with the smallest confidence interval in line with the retroactive waiver, meeting a minimum quality standard for the model was a pre-condition of the option to be selected.  It may be that the econometric model is better, but it is also arguable that it would be of a higher quality without the scaled spillover variable, and the Verification Report should consider reformulating the model without such a problematic variable.

##### CONFORMITY WITH THE PROTOCOLS

Measurement Protocols:  This Study appears to be in general conformity with the retroactive waivers to the measurement Protocols.

- Reporting Protocols:  Tables 6 and 7 are adequately documented, although Table 7 frequently references whole sections of the study instead of summarizing the points, as Table 7 was intended to do.

##### RECOMMENDATION

Pending the result of the Verification Report which should investigate the integrity of the spillover analysis, the recommendation is to disallow the spillover claims in the load impact Study.

As reported in Table 6 of the Study

We will assume that this does not mean literally the lowest bound of the precision interval, but rather the model that produces the smallest or tightest confidence interval at a given level of precision.