Verification  Report - 2001 AEAP

ECONorthwest

888 SW Fifth Ave. - Suite 1460

Portland, Oregon 97204

(503) 222-6060

(503) 222-1504 (fax)

Pacific Gas & Electric Company – Study No. 372R1

Evaluation of Pacific Gas & Electric Company’s 1996 Appliance Energy Efficiency Incentives Program Third-Year Retention Study

Table of Contents

Introduction and Executive Summary	1

Measures Studied	2

Methodologies	2

Summary of Findings	2

Recommendation to ORA	3

Data and Documentation Quality	3

Data	3

Documentation	3

Replication and Analysis	3

Review of Analytic Approach and Dataflow	3

Replication Efforts	5

Review of Database Development	5

Review of Analytic Procedures	5

Modifications to Database and Analytical Procedures	5

Database Modifications	5

Analysis Modifications	5

Recommended Changes to Filed EUL Estimates	5

- PG&E - Study No. 372R1

1996 Appliance Energy Efficiency Incentives Program Third-Year Retention Study

# Introduction and Executive Summary

This is a Verification Report (“VR”) of Pacific Gas & Electric Company’s (“PG&E”) retention study for residential lighting measures for which rebates were paid during the 1996 program year (“PY96”) through PG&E’s Residential Appliance Efficiency Incentives (“RAEI”) Program.  This study was performed by XENERGY.

This VR is presented in five sections.  The first section contains this introduction and the executive summary of the findings, along with the recommendations to the Office of Ratepayers Advocates (“ORA”).  The second section discusses the data and documentation supplied by PG&E and XENERGY to support the study.  The third section details ECONorthwest’s replication and assessment of the analytical procedures and corresponding SAS code used in the study.  The fourth section reports recommended modifications to the dataflow and analytical procedures used in the study.  The final section presents the recommended changes to the filed effective useful life (EUL) calculations for each measure studied.

The study reports estimates of the EUL for residential lighting measures using data collected on the fraction of installed measures in place and operable.  The EUL for each measure is calculated by estimating the median number of years that the measure is still in place and operable from modeled survival functions.  Ex post EUL estimates are then compared with ex ante estimates at the 80 percent confidence level.

ECONorthwest’s verification efforts include:

•	evaluation of the study methodology;

•	replication of the statistical findings of the study; and

•	recommendations to the ORA.

## Measures Studied

The Protocols require that the utilities conduct a retention study on “the top ten measures, excluding measures that have been identified as miscellaneous (per Table C-9), ranked by net resource value or the number of measures that constitutes the first 50% of the estimated resource value, whichever number of measures is less.”  The study looks at compact fluorescent lights (“CFLs”), high intensity discharge fixtures (“HIDs”), and T-8 lighting fixtures and ballasts which together constitute 87 percent of the total kWh impacts in the RAEI Program’s Residential Lighting component for PY96.  On-site inspections were conducted at 57 sites to gather measure retention data for use in the survival analysis.  Overall, at the time of the inspections, approximately only 7 percent of the sampled measures had experienced failure.

## Methodologies

The analysis techniques employed in the study consist of collecting measure retention data from a sample of participants, and fitting a parametric survival function to the retention data using classic survival analysis techniques.  The following parametric forms of the hazard function are used to calculate alternative survival functions and generate estimates of the EUL for each measure with observed failures:

•	Gamma

•	Weibull

•	Exponential

•	Log-normal

•	Log-logistic

Unfortunately, the exponential survival function was the only model that converged for all three measures studied.  This functional form, however, does not fit the expected failure pattern for those measures studied.

The study also included an analysis of within- and between-site variance for use in calculating the standard error associated with each estimated EUL.  This analysis was conducted by calculating values of rho and the design effects factor, using program data.  This analytical step provides valuable results that have not been previously utilized in retention studies.  ECONorthwest applauds this effort.

## Summary of Findings

At the time of the on-site surveys, there had been an insufficient number of failures to determine the appropriate model form or to estimate the EUL with the statistical precision required at the 80 percent confidence level.  XENERGY, therefore, recommends that no adjustments be made to the ex ante EUL estimates for those measures studied.

XENERGY’s method for adjusting the EUL standard error estimates by the square root of the design effects factor is appropriate and addresses issues raised by ECONorthwest in its review of XENERGY’s, and others’, retention studies during the 1999 AEAP.

## Recommendation to ORA

ECONorthwest recommends that no adjustments be made to the ex ante EUL for those measures studied.

# Data and Documentation Quality

## Data

Files were provided on one compact disk and ECONorthwest encountered no problems with any aspect of PG&E and XENERGY’s provision of data. The majority of XENERGY’s analysis is performed in SAS.  Microsoft Access database software is also used in XENERGY’s initial preparation of the analytic dataset.

## Documentation

The study provided helpful documentation.  A thorough description of the methodology and helpful exhibits were included to assist with the replication effort.

# Replication and Analysis

## Review of Analytic Approach and Dataflow

The study uses classic survival analysis techniques to estimate the EUL of each measure from observed failures.  The PROC LIFEREG procedure in SAS is applied to the measure retention data to obtain estimates of the EUL under five alternative parametric forms of the hazard function.

The study adjusts the standard error and confidence interval estimates for each model by dividing the unadjusted standard errors on the EUL estimates by the square root of the design effect factor. The design effect factor measures the extent to which non-retention occurs at the project or measure level.  The design effect factor for each measure is derived by first calculating rho, an estimate of intra-cluster correlation, and then np, the average number of expected units of a measure per sample project.

The hazard function represents the instantaneous failure rate for an installed measure that has survived to a particular age.  The five parametric forms of the hazard function considered in the study exhibit the following characteristics:

- Gamma Model: The gamma modeling assumption is the most general of the five distributions considered.  It allows for both the estimation of the rate of change (scale) and the change in rate (shape) of the hazard function.  Because both scale and shape parameters can be estimated, the gamma model results in the best functional fit relative to the other distributions examined in the study.  The hazard associated with the gamma model can take on a variety of shapes depending on the value of the scale and shape parameters.  Unlike the other hazard distributions used in the study, the gamma model’s hazard function can take the form of a U, or bathtub shape, in which the hazard initially decreases with time and later increases.

- Weibull Model: The Weibull model is a proportional hazard model that allows a scale parameter to be estimated.  When the scale parameter is less than one, the Weibull’s hazard function increases with time.  When the scale parameter is greater than one, the resulting hazard function decreases with time.

- Exponential Model: The exponential model is the most restrictive of the models and does not allow for the estimation of shape or scale parameters.  The exponential assumption assumes a constant hazard function and is equivalent to the Weibull model with a scale parameter value of one.

- Log-normal Model: The log-normal model assumes that the hazard function is non-monotonic.  The hazard starts at zero, rises to a peak, and then declines towards zero.  A scale parameter is estimated when using this model.

- Logistic Model: The logistic model allows for the estimation of a scale parameter.  It also allows for, but does not assume a non-monotonic hazard function.  For a scale parameter less than one, the log-logistic hazard function resembles the log-normal hazard function.  When the scale parameter is greater than one the hazard function starts at infinity and declines towards zero with time.

In general, one would expect that the true hazard function for most measures would eventually increase over time.  Both the gamma and Weibull models allow for the estimation of a survival function that exhibits this property.  In practice, we find that the gamma and Weibull models generally result in more realistic EUL estimates for most measures.

The retention database used in the study contains data collected during on-site surveys on installed measures from sites included in the PY96 retention panels.  In some cases, the exact removal or failure date of a particular installed measure was unknown, resulting in left censoring.  However, the majority of observations in the retention database had not failed at the time of the survey, thus most values were right censored.  The SAS procedure, PROC LIFEREG, can accommodate left, right, and interval censored values.

In summary, the study’s approach is a sound and useful approach.  ECONorthwest is particularly pleased to see that XENERGY has addressed issues raised during the 1999 AEAP in our review of study 311R1, 328R1, 314R1, and 325R1.

## Replication Efforts

The verification effort included a review of programming code for errors; comparison of code steps with methodological descriptions contained in the report or accompanying documentation; and reconstruction of the analytical results accomplished by running the programming code.  ECONorthwest paid particular attention to the theoretical appropriateness of the methodologies employed.

### Review of Database Development

Although most of the verification effort focused on other aspects of the study, ECONorthwest did not encounter any problems when reviewing the database development processes used in this study.

### Review of Analytic Procedures

The analysis proceeded as described in the study and was in general compliance with the M&E Protocols.  Unfortunately, not enough time as passed since the 1996 program year to obtain statistically reliable EUL estimates for the lighting measures studied.

# Modifications to Database and Analytical Procedures

## Database Modifications

No modifications are recommended for the database procedures used in this study.

## Analysis Modifications

The analytical technique employed in the study is a sound and useful technique.  No modifications are recommended for the analytical portion of this study.

# Recommended Changes to Filed EUL Estimates

ECONorthwest recommends that no adjustments be made to the ex ante EULs for those measures studied.

“Protocols and Procedures for the Verification of Costs, Benefits, and Shareholder Earnings from Demand-Side Management Programs,” as adopted by California Public Utilities Commission Decision 93-05-063, Revised March 1998.