Verification  Report - 2001 AEAP

ECONorthwest

888 SW Fifth Ave. - Suite 1460

Portland, Oregon 97204

(503) 222-6060

(503) 222-1504 (fax)

Pacific Gas & Electric Company – Study No. 373 1R1

Evaluation of Pacific Gas & Electric Company’s 1996 - 1997 Appliance Energy Efficiency Programs: Residential Refrigeration Fourth Year Retention Study

Table of Contents

Introduction and Executive Summary	1

Measures Studied	2

Methodologies	2

Summary of Findings	3

Recommendation to ORA	3

Data and Documentation Quality	3

Data	3

Documentation	3

Replication and Analysis	3

Review of Analytic Approach and Dataflow	3

Replication Efforts	5

Review of Database Development	5

Review of Analytic Procedures	5

Modifications to Database and Analytical Procedures	5

Database Modifications	5

Analysis Modifications	6

Recommended Changes to Filed EUL Estimates	6

- PG&E - Study No. 373 1R1

1996 - 1997 Appliance Energy Efficiency Programs: Residential Refrigeration Fourth-Year Retention Study

# Introduction and Executive Summary

This is a Verification Report (“VR”) of Pacific Gas & Electric Company’s (“PG&E”) retention study for energy efficiency refrigeration measures for which rebates were paid during the 1996 and 1997 program years (PY96 and PY97) through PG&E’s Residential Appliance Efficiency Incentives (“RAEI”) Program.  This study was performed by Quantum.

This VR is presented in five sections.  The first section contains this introduction and the executive summary of the findings, along with the recommendations to the Office of Ratepayers Advocates (“ORA”).  The second section discusses the data and documentation supplied by PG&E and Quantum to support the study.  The third section details ECONorthwest’s replication and assessment of the analytical procedures and corresponding SAS code used in the study.  The fourth section reports recommended modifications to the dataflow and analytical procedures used in the study.  The final section presents the recommended changes to the filed effective useful life (“EUL”) calculations for energy efficiency refrigeration units.

The study reports estimates of the EUL for residential energy efficient refrigerators using data collected on the fraction of installed measures in place and operable.  In addition, data was collected on old refrigeration units, which was incorporated into the survival analysis.  The EUL for the energy efficiency refrigerators is calculated by estimating the median number of years that the measure is still in place and operable from modeled survival functions.  Ex post EUL estimates are then compared with ex ante estimates at the 80 percent confidence level. The study included the following analysis techniques:

- compilation of summary statistics on the raw, measure retention data;

- visual inspection of the measure retention data;

- development of a trend line from the survival plots;

- development of a survival function using classic survival analysis techniques; and

- development of a competing risk models.

ECONorthwest’s verification efforts include:

- evaluation of the study methodology;

- replication of the statistical findings of the study; and

- recommendations to the ORA.

## Measures Studied

The Protocols require that high efficiency refrigeration measures be studied for the RAEI program.  The study focuses on residential refrigeration measures for which rebates were paid during PY96 and PY97.  Out of the 589 completed surveys, only one failure and nine removals were observed.  These low failure counts are attributed to the fact that energy efficient refrigerators have an ex ante EUL of 20 years; the data for this study was collected after only three or four years.  Given the limited time frame and relatively few failures, reliable EUL estimates from the retention data are not possible.

In addition to collecting data for new, energy efficient refrigerators, the study also utilizes measure retention data for old refrigerators that spans a much longer time period (15years).  An ex post EUL is then estimated and used to substantiate the ex ante EUL of 20 years for these older, energy efficient refrigerators.

## Methodologies

The analysis techniques employed in the study consists of five analytical steps:

- The initial step consisted of compiling summary statistics from the raw retention data.

- The next step involved plotting the cumulative measure retention data over time.

- Linear and exponential trend lines are then generated from the cumulative measure retention data.

- The measure retention data was fitted to alternative parametric survival functions using classic survival analysis techniques.  The following survival distributions were explored for both old refrigerators and newer, energy efficient refrigerators: exponential, logistic, log-normal, Weibull, and gamma.

- Lastly, a competing risk model was developed which incorporates different survival distributions for failures, removals, and replacements.

## Summary of Findings

The EUL estimates varied significantly across models, with large standard errors and confidence intervals on EUL estimates.  This is primarily attributed to the relatively short time span between the installation date and the survey date for this measure.  As with most of the third year retention studies, little confidence can be placed on the EUL estimates generated in the study because they are based only on data from the first few years after installation for measures that are expected to last, in this case, 20 years.  Quantum does a good job of discussing the difficulty and limitations of conducting a retention study in the third or fourth year for measures with such a lengthy, expected useful life.  In summary, the methodology used by Quantum is adequate and would likely produce statistically robust results if the retention study was conducted after more years had passed.

The analysis performed using measure retention data for old refrigeration measures, indicates that these units have an EUL of approximately 19 years.  This result is used to validate the ex ante EUL of 20 years for these older, energy efficient refrigerators.

## Recommendation to ORA

ECONorthwest recommends that no adjustments be made to the ex ante EUL as documented in the study.

# Data and Documentation Quality

## Data

Files were provided on one compact disk and ECONorthwest encountered no problems with any aspect of PG&E and Quantum’s provision of data.  The majority of Quantum’s retention analysis was performed in SAS.

## Documentation

The study included sufficiently detailed documentation.  Indeed, a thorough description of the methodology and helpful exhibits were included to assist with the replication and assessment effort.

# Replication and Analysis

## Review of Analytic Approach and Dataflow

The study employs four analytical steps before deriving ex post EUL estimates with competing risk models.  The first step consisted of compiling summary statistics for both refrigerator measures based on collected survey data compiled in the retention database. Next, the empirical survival distributions are plotted, and exponential and linear trend lines are developed for both types of refrigeration units.

The study continues by using classic survival analysis techniques to estimate each measure’s ex post EUL. The PROC LIFEREG procedure in SAS is applied to the retention data to obtain estimates of the EUL under five alternative, parametric forms of the hazard function.  The hazard function represents the instantaneous failure rate for an installed measure that has survived to a particular age.  The five parametric forms of the hazard function considered in the study exhibit the following characteristics:

- Gamma Model: The gamma modeling assumption is the most general of the five distributions considered.  It allows for both the estimation of the rate of change (scale) and the change in rate (shape) of the hazard function.  Because both scale and shape parameters can be estimated, the gamma model results in the best functional fit relative to the other distributions examined in the study.  The hazard function associated with the gamma model can take on a variety of shapes depending on the value of the scale and shape parameters.  Unlike the other hazard distributions used in the study, the gamma model’s hazard function can take the form of a U, or bathtub shape, in which the hazard initially decreases with time and later increases.

- Weibull Model: The Weibull model is a proportional hazard model that allows a scale parameter to be estimated.  When the scale parameter is less than one, the Weibull’s hazard function increases with time.  When the scale parameter is greater than one, the resulting hazard function decreases with time.

- Exponential Model: The exponential model is the most restrictive of the models and does not allow for the estimation of shape or scale parameters.  The exponential assumption assumes a constant hazard function and is equivalent to the Weibull model with a scale parameter value of one.

- Log-normal Model: The log-normal model assumes that the hazard function is non-monotonic.  The hazard starts at zero, rises to a peak, and then declines towards zero.  A scale parameter is estimated when using this model.

- Logistic Model: The logistic model allows for the estimation of a scale parameter.  It also allows for, but does not assume a non-monotonic hazard function.  For a scale parameter less than one, the log-logistic hazard function resembles the log-normal hazard function.  When the scale parameter is greater than one, the hazard function starts at infinity and declines towards zero with time.

In general, one would expect that the true hazard function for most measures would eventually increase over time.  Both the gamma and Weibull models allow for the estimation of a survival function that exhibits this property.  In practice, we find that the gamma and Weibull models generally result in more realistic EUL estimates for most measures.

The final analytical step in the survival analysis involves developing a competing risk model.  Survival estimates are generated individually for failures and removals for energy efficient refrigeration units.

The best fitting survival distribution for each of these events is then determined by identifying the distribution that has the greatest log-likelihood statistic associated with it.  The best fitting survival functions for each measure and each of these events are then combined to form a single, combined survival function which is then used to estimate the EUL.  In addition, a second, competing risk model is developed using the failure and removal data obtained during the data collection process for the old refrigerators.

The retention database used in the study contains data collected during on-site surveys of installed measures at sites included in the PY96 and PY97 retention panels.  In some cases, the exact removal or failure date of a particular installed measure was unknown, resulting in left censoring.  However, the majority of observations in the retention database had not failed at the time of the survey, thus most values are right censored.  The SAS procedure, PROC LIFEREG, can accommodate left, right, and interval censored values.

In summary, the study’s approach is a sound and useful approach.  Unfortunately, it is apparent from the lack of failures that the fourth-year retention/survival study for energy efficient refrigerators is premature.

## Replication Efforts

The verification effort included a review of programming code for errors; comparison of code steps with methodological descriptions contained in the report or accompanying documentation; and a partial reconstruction of the analytical results accomplished by running several sections of programming code.  ECONorthwest paid particular attention to the theoretical appropriateness of the methodologies employed.

### Review of Database Development

Although most of the verification effort focused on other aspects of the study, ECONorthwest did not encounter any problems when reviewing the database development processed used in this study.

### Review of Analytic Procedures

The analysis proceeded as described in the study and was in general compliance with the M&E Protocols.  Due to the lack of observed failures, the EUL estimates, in general, varied significantly across hazard distributions with standard errors that were quite large.  The study correctly concludes that not enough time has passed to accurately model the EUL from the retention data collected for energy efficient refrigerators.  The use of failure data for old refrigeration units is very helpful and adds significantly to the analysis performed in the study.

# Modifications to Database and Analytical Procedures

## Database Modifications

No modifications are recommended for the database procedures used in this study.

## Analysis Modifications

The analytical technique employed in the study is a sound and useful technique.  No modifications are recommended for the analytical portion of this study.

# Recommended Changes to Filed EUL Estimates

ECONorthwest recommends that no adjustments be made to the ex ante EUL as documented in the study.