Verification  Report - 2001 AEAP

ECONorthwest

888 SW Fifth Ave. - Suite 1460

Portland, Oregon 97204

(503) 222-6060

(503) 222-1504 (fax)

San Diego Gas & Electric Company – Study No. 984

Evaluation of San Diego Gas & Electric Company’s 1996 – 1997 Residential Appliance Efficiency Incentives Program (Compact Fluorescent Lights) Fourth-Year Retention Study

Table of Contents

Introduction and Executive Summary	1

Measures Studied	1

Methodology	2

Summary of Findings	2

Recommendation to ORA	2

Data and Documentation Quality	3

Data	3

Documentation	3

Replication and Analysis	3

Review of Analytic Approach and Dataflow	3

Review of Database Development	3

Review of Analytic Procedures	3

Modifications to Database and Analytical Procedures	4

Database Modification	4

Analysis Modifications	4

Recommended Changes to Filed EUL Estimates	4

- SDG&E - Study No. 984

1996 – 1997 Residential Appliance Efficiency Incentives Program (Compact Fluorescent Lights)   Fourth-Year Retention Study

# Introduction and Executive Summary

This is a Verification Report (VR) of San Diego Gas & Electric Company’s (SDG&E) retention study for compact fluorescent lighting measures (CFLs) installed as part of their Residential Appliance Efficiency Incentives Program (RAEI) during the 1996 and 1997 program years (PY96 and PY97).  Telephone surveys for this study were conducted by CIC Research, Inc.

This VR is presented in five sections.  The first section contains an introduction and executive summary of the findings, along with recommendations to the Office of Ratepayers Advocates (ORA).  The second section discusses the data and documentation supplied by SDG&E and audit contractors.  The third section details ECONorthwest’s replication and assessment of the analytical procedures used in the study.  The fourth section reports recommended modifications to the dataflow and analytical procedures used in the study.  The final section presents the recommended changes to the filed effective useful life (EUL) estimates for each measure studied.

Based on a random sample of program participants, the study reports estimates of the EUL for compact fluorescent lighting measures (CFLs) using data collected on the fraction of installed measures in place and operable.  The EUL for each measure is calculated by estimating the median number of years that the measure is still in place and operable from modeled survival functions.  Ex post EUL estimates are compared with ex ante estimates at the 80 percent confidence level to calculate realization rates.

ECONorthwest’s verification efforts include:

- evaluation of the study methodology,

- replication of the statistical findings of the study, and

- recommendations to the ORA.

## Measures Studied

The Protocols require that the utilities conduct a retention study of “the top ten measures, excluding measures that have been identified as miscellaneous (per Table C-9), ranked by net resource value or the number of measures that constitutes the first 50% of the estimated resource value, whichever number of measures is less.” In PY96 and PY97, the RAEI Program offered incentives on both CFL bulbs and fixtures.  CFL measures of various watt usage were issued through several distribution methods, however, all types of bulbs and fixtures in the RAEI Program were surveyed regardless of physical attributes or distribution methods.  As such, there are no “like” measures in this program.  The ex ante EUL for CFL bulbs is eight years for PY96 and nine years for PY97, and the ex ante EUL estimate for CFL fixtures is 20 years in both program years.

## Methodology

The analysis techniques employed in the study consist of collecting measure retention data from program participants and, given the dichotomous nature of this data, uses classical survival analysis to produce a revised estimate of the effective useful life of energy efficiency measures installed under this program.  The revised EUL estimate (ex post EUL) is then compared to the forecast EUL (ex ante EUL) to derive the EUL realization rate.  If this difference is not statistically significant, then the forecast estimate is used to calculate resource benefits and earnings in the utility’s third and fourth earnings claims.

## Summary of Findings

SDG&E’s RAEI retention study evaluated two CFL measures.  The highlights from ECONorthwest’s verification efforts are:

- The statistically significant ex post EUL for CFL fixture measures is estimated at 17.2 years.  This yields a realization rate of 0.86 for these measures.  SDG&E recommends that this ex post EUL be used as the basis for their third and fourth year earnings claim.

- The ex post EUL for CFL bulb measures was found to be statistically significantly different than the ex ante values at the 80 percent confidence level and SDG&E, therefore, recommends in Table 6 of the retention study that the ex post EUL be used as the basis for their third and fourth year earnings claim.  The realization rate for this measure is 0.76.

## Recommendation to ORA

ECONorthwest recommends that the ex post EUL for CFL bulbs and fixtures be accepted as the basis for SDG&E’s third and fourth earnings claim for this program.

# Data and Documentation Quality

## Data

Files were provided on one compact disc and ECONorthwest encountered no problems with SDG&E’s data.  Although the retention survey data was not included for review, the results from those surveys were included in the individual measure models.  Given the complexity of the modeling spreadsheets, ECONorthwest opted to focus its verification efforts on those spreadsheets.

## Documentation

The study itself was adequately documented.  Although the description of the methodology was brief, SDG&E included a completed Table 6 that clearly reported the findings of the retention study, and a fully articulated Table 7 that assisted the verification effort.  In addition, SDG&E provided supplemental documentation that facilitated our review of the two measure models.  The measure models were quite sophisticated and programmed into individual Excel workbooks, thus, this supplemental documentation was crucial for our verification efforts.

# Replication and Analysis

## Review of Analytic Approach and Dataflow

The primary features of the retention model used to estimate the EUL for energy efficiency measures installed as part of the RAEI-CFL Program are the specification of a hazard function and its associated survival function, the estimation of the parameters of that survival function using a maximum likelihood estimation methodology, and the estimation of a median expected lifetime.  ECONorthwest reviewed the methodology employed in the retention study, as well as its actual implementation within the spreadsheet models.

### Review of Database Development

ECONorthwest did not review the development of the retention database used for this study.  The results from the retention study surveys (i.e., the retention database), however, were included in the individual measure retention models.  ECONorthwest verified that these survey results matched those reported in Table 7 of the report.

### Review of Analytic Procedures

ECONorthwest reviewed both the general approach of the analysis, as well as the specific implementation of analytical procedures within each of the spreadsheet (Excel) models developed for the two lighting measures included in the retention study.  The general approach of the analysis appears reasonable.  The quadratic specification of the hazard function employed in the study is useful.  However, ECONorthwest recommends that other functional forms be investigated and discussed in future studies.

The study relies on maximum likelihood estimation of the parameter values that are used in the median lifetime calculations for individual measures.  Maximum likelihood estimators (MLEs) are, typically, good estimators, i.e, best linear unbiased estimators.  In addition, the use of the log-likelihood function, as was done in the study, is desirable for many reasons—particularly because the logarithmic function is monotonic  and the estimated parameters are constant. Although there are many computer programs that perform the necessary calculations, the study authors, interestingly, programmed Excel spreadsheets with exponential and linear grid search routines.

As part of verifying that the underlying mathematics were correctly programmed into each measure spreadsheet, ECONorthwest reviewed the linkages, formulae, and macros used to calculate EUL estimates and their associated standard errors.  In addition, ECONorthwest made sure that the quantity and date (if known) of measure failures contained in the spreadsheet model matched those reported in Table 7 of the retention study.  Finally, ECONorthwest ensured that the upper and lower bounds of the confidence interval were correctly calculated in Table 6.

# Modifications to Database and Analytical Procedures

## Database Modification

Since the retention database was not reviewed, no modifications are recommended for the database portion of the retention study.

## Analysis Modifications

The overall approach and specific programming involved in the retention models was sound, thus, ECONorthwest recommends no changes to the analytic elements of this retention study.

# Recommended Changes to Filed EUL Estimates

The ex post EUL estimates for both CFL lighting measures were found to be statistically significantly different than SDG&E’s ex ante EUL estimates used in the first and second earnings claim.  SDG&E, therefore, recommends in Table 6 of the retention study that the ex post EUL for each of these measures be adopted for the third and fourth earnings claim.

ECONorthwest recommends that the ex post EULs for CFL bulb and fixture measures be accepted as the basis for SDG&E’s third and fourth earnings claim for this program.

“Protocols and Procedures for the Verification of Costs, Benefits, and Shareholder Earnings from Demand-Side Management Programs,” as adopted by California Public Utilities Commission Decision 93-05-063, Revised March 1998.

This result is based on the exclusion of a single site where all 100 fixtures were reported out of service.  Inclusion of this site in the retention sample would yield a statistically significant ex post EUL of 15.2 years.

As discussed previously, the ex ante EUL for CFL bulbs is 8 years in PY96 and nine years in PY97.  SDG&E, therefore, reports the ex ante EUL for lighting measures in Table 6 as 8.5 years.

Much of this documentation was supplied during the 1999 AEAP.

The estimated variance, however, is a biased estimator of the actual variance.  Although biased, the estimated variance is consistent.  See Pindyck and Rubinfeld, “Econometric Models and Economic Forecasts,” pg. 71.

Modeling in Excel denied the authors the alternative of using more advanced statistical techniques available in, for instance, SAS.  In the case of left hand censoring that arises when the failure date of the measure is unknown and is, rather arbitrarily, set as the month of the survey, SDG&E and its consultant were unable to rely on the LIFEREG and PHREG procedures in SAS.